5/29/2024

Got together with the group. We started talking about the basics of kite design but did nothing. Got meeting times down.

5/30/2024

Made a plan for the coming weeks. Planned the first prototype. The idea is to glue together some dowels and trash bag film to make a delta kite, then test it.

6/05/2024

- We connected the servo to gemma mo and used arduino IDE to create basic servo movement

- To set up arduino IDE for gemma mo:

- Go to IDE settings and type in https://adafruit.github.io/arduino-board-index/package_adafruit_index.json in the Additional boards manager URLs. Click OK.

- Go to Tools>Board>Board Managers> and install Adafruit SAMD Boards

- Then go back to Tools> Board>Adafruit SAMD boards> choose Adafruit Gemma Mo

- Then go to Programmer> choose Atmel-ICE

- Then go to Port and choose the appropriate port where the Gemma is connected

- Start coding> press upload

- Parts to design:

- We need to start designing a connecter that holds connects the servo to the wings’ spar so that we can move the wings up and down

- We also need to design a holder for the servo that will sit on the middle spar

06/06/2024

06/07/2024

Channel Values for Spektrum Controller

|  | -a (min) | 0 | +a (max) |
| --- | --- | --- | --- |
| Thot (2) | 1098 | 1468 | 1880 |
| Aile (3) | 1777 | 1470 | 1191` |
| Elev (4) | 1317 | 1605 | 1895 |
| Budder (5) | 1780 | 1490 | 1200 |

6/10/24

Sai Retest

Min/Max

|  | -a (min) | 0 | +a (max) |
| --- | --- | --- | --- |
| Battery (2) | — | — | — |
| Thot (3) | 1084 | 1468 | 1893 |
| Aile (4) | 1844 | 1470 | 1039 |
| Elev (5) | 1207 | 1605 | 2000 |
| Budder (6) | 1950 | 1490 | 1144 |

Actual Control (we want to create a range where nothing happens)

| Flap frequency | -a (min) | 0 | +a (max) |
| --- | --- | --- | --- |
| Battery (2) |  |  |  |
| Thot (3) | 1184 |  | 1793 |
| Aile (4) | 1744 |  | 1139 |
| Elev (5) | 1307 |  | 1900 |
| Budder (6) | 1850 |  | 1244 |

6/27

Got the high torque servo working. It accepts a range of 700 to 2294 instead of the theoretical 700-2300 but yeet. Even more theoretical the adafruit docs say it should be 1000-2000 but at this point field robotics does as field robotics does.